# Safety performance Indicators (SPIs)

- Definition: Safety Performance Indicators (SPIs) are

- measurable metrics used to monitor and assess an organization's safety performance,

- helping to verify safety goals,

- identify trends,

- detect risks, and

- check SMS effectiveness through data like incident rates, audit findings, and training compliance

- Types

- Quantitative and Qualitative

- Qualitative: descriptive e.g. employee perception of safety culture

- Quantitative:

- expressed as number (e.g. 10 incursions)

- expressed as rate (e.g. 10 incursions per 1000 movement)

- Leading and lagging

- Lagging:

- SPIs that have already occurred

- Outcome based

- Reactive

- Normally negative outcomes organization aim to avoid

- Types:

  - Low probability/high severity (lagging SPIs): accidents or serious incidents e.g. aircraft damage due to bird strike

  - High probability/low severity (Precursor SPIS): help detect risk building up before an accident happens e.g. bird radar detection which indicates level of bird activity rather than actual bird strikes

- Leading

- Measures process and inputs

- Activity or process SPIs

- Proactive

- Monitors and measures condition that have potential to lead or to contribute to a specific outcome

- E.g. Percentage of staff who have successfully completed safety training on time

- E.g. frequency of bird scaring activities

- Link between leading, precursor and lagging SPIs

For Bird hazard management:

- Leading SPI: Bird scaring activities (Actions)

- Precursor SPI: Bird sightings or radar detections (Warning)

- Lagging SPI: No. of bird strikes (Outcome)

- Note: If leading activities decrease, precursor spikes and lagging incidents increase.

Sequence of defining: Lagging  Precursor  Leading

Note: The only real mistake is the one from which we learn nothing. – John Powell

- Selecting SPI

SPIs should focus on parameters that are important indicators of safety performance, rather than on those that are easy to attain.

SPIs should be:

a) related to the safety objective they aim to indicate;

b) selected or developed based on available data and reliable measurement;

c) appropriately specific and quantifiable; and

d) realistic, by taking into account the possibilities and constraints of the organization.

A combination of SPIs is usually required to provide a clear indication of safety performance. There should be a clear link between lagging and leading SPIs. Ideally lagging SPIs should be defined before determining leading SPIs.

- Defining SPIs

The contents of each SPI should include:

a) a description of what the SPI measures;

b) the purpose of the SPI (what it is intended to manage and who it is intended to inform);

c) the units of measurement and any requirements for its calculation;

d) who is responsible for collecting, validating, monitoring, reporting and acting on the SPI (these may be staff from different parts of the organization);

where or how the data should be collected; and

f) the frequency of reporting, collecting, monitoring and analysis of the SPI data.

- SPIs and safety reporting

In simple terms,reporting bias happens when the number of safety reports goes up or down for the wrong reasons, making data less accurate:

Underreporting: People may stop reporting hazards if they are confused by new rules or don't trust them yet.

Over-reporting: People might report too much if new laws give them extra protection or incentives.

The Bottom Line: Even though these biases can "mess up" the data used to measure safety, reports are still very useful as long as managers look at them carefully and understand the context.

This document contains monitoring report of SPI with graphs, table and summary.